Auxiliary components and accesories

Bearings

Two families:

- Rolling-element bearings (ball/roller) — used in aero-derivative gas turbines with light rotors (e.g., LM5000 rotor ~1,230 lb). No need for large lube-oil reservoirs/pre-post-lube cycles. Balls = lower capacity, higher speed; rollers = higher capacity, lower speed.

- Journal (fluid-film/sleeve) bearings — used in nearly all heavy-frame industrial gas turbines. Oil film completely separates journal from bushing; hydrodynamic pressure balances the load.

Journal bearing types (know these — classic interview list), roughly in order of increasing stability/cost:

- Plain journal — simplest, uniform clearance

- Circumferential grooved — better cooling, less capacity

- Cylindrical bore — split, two axial feed grooves

- Pressure dam — plain bore + a dam that forces a hydrodynamic wedge; one direction only

- Lemon bore/elliptical — bidirectional

- Three-lobe — rare in turbomachinery

- Offset halves — bidirectional-restricted, good stiffness

- Tilting-pad — most common in modern machines; self-aligning, best resistance to half-speed (oil) whirl, but adds parasitic power loss and needs more oil flow

Selection drivers: shaft speed, misalignment tolerance, critical-speed/stiffness interaction, impeller loading, oil temp/viscosity, foundation stiffness, allowable axial movement, contamination, vibration limits.

Thrust bearings handle axial loads — separate chapter subsection, tied closely to the single- vs double-helical gear discussion below.

Seals

Two big categories:

Non-contacting (clearance) seals

- Labyrinth seals — cheapest/simplest, tolerant of dirt and thermal excursions, but leak more and can't meet OSHA/EPA emissions limits. Leakage ∝ 1/√(number of lands). Used for interstage seals, shroud seals, balance pistons, static case joints.

- Windback seals — screw-thread device that "pumps" oil back using shaft windage; often paired with other seals to keep oil off carbon faces.

- Ring (bushing) seals — fixed or floating; floating is far more common (moves with the shaft, avoids wear from rubbing). Carbon rings used when sealing gas/air (self-lubricating).

Contacting: Mechanical seals — selection depends on Product (chemistry, pressure, temp, lubricity, abrasion, corrosion, toxicity), Seal Environment (flushing, barrier fluid, quench, heating/cooling), Seal Arrangement (single/double, balanced/unbalanced), and Equipment.

- Balanced vs unbalanced: unbalanced good to ~100 psig; above that, use balanced.

- PV rating (face load × sliding velocity) caps design: ~200,000 unbalanced, ~2,250,000 balanced.

Dry gas seals — this is the one to know cold; >85% of centrifugal gas compressors now use them, replacing oil-film seals. Non-contacting mechanical face seal: rotating mating ring with spiral grooves + stationary primary ring. Sealing gas is sheared inward by the grooves, builds hydrostatic/hydrodynamic pressure, lifts the primary ring off — running gap ~3 microns. Arrangements:

- Tandem — second seal on atmosphere side acts as backup/safety seal, vents to flare

- Tandem with labyrinth — used when even trace product leakage or buffer-gas-into-process is unacceptable (H₂, ethylene)

- Double gas seal — buffer gas fed between two seals at pressure above process gas

Operating envelope: up to ~2,400 psi, 500°F, 500 ft/s surface speed. Materials: tungsten/silicon carbide faces, Viton/Kalrez O-rings, 300/400 SS hardware.

Gears (Ch. 14)

- Single- vs double-helical: double-helical self-cancels axial thrust (still often gets a thrust bearing per API 613/617 recommendation); single-helical requires a thrust bearing per API and has asymmetric loading that oscillates the pressure center — a classic bearing-failure root cause.

- Design factors: pressure angle (17.5°–22.5° typical, higher = more tooth strength but more bearing load), helix angle (5°–45°), tooth hardness, gear accuracy, service factor, housing rigidity, lubrication.

- Lubrication: only 10–30% of oil does actual lubrication — 70–90% is pure cooling. Oil sprayed into the "outmesh" (disengaging side) up to ~15,000 ft/min pitch-line speed; above that, split spray outmesh/inmesh.

- Manufacturing: hobbing (generates tooth form, ~40 μin finish limit) → shaving or grinding for high-speed gearing finish.

- Failure modes: root bending fatigue, case crushing (insufficient case depth/soft core) — referenced against ANSI/AGMA 1010-E95.

A couple of practical notes for the interview:

- If asked "what standard governs seal/gear selection in this industry," API 613/617 is the answer for rotating equipment (compressors, gear units) and AGMA standards are the gear-specific design/manufacturing/accuracy standards layered on top.

- The handbook itself doesn't cite bearing-specific API standards in this excerpt (e.g., API 610 for pumps, API 670 for machinery protection systems aren't mentioned here) — if GE Vernova's interview touches instrumentation/vibration monitoring, you may want to know API 670 exists even though it's outside this chapter's scope. Want me to search for a quick rundown of the broader API standards family relevant to rotating machinery, in case it comes up beyond what's in this PDF?

Summary

Bearings

- Rolling-element bearings dominate light aero-derivative rotors; journal (fluid-film) bearings dominate heavy industrial frames.

- Tilting-pad journal bearings are the modern standard — best resistance to oil whirl, self-aligning, but higher cost and parasitic power loss.

- Bearing selection is driven by speed, misalignment, critical speed/stiffness, oil viscosity, and vibration tolerance — not just load.

Seals

- Labyrinth seals: simple, robust, cheap — but leaky and can't meet modern emissions rules (OSHA/EPA pressure pushed the industry away from them).

- Floating ring seals beat fixed rings because they track shaft motion instead of rubbing.

- Mechanical seal selection hinges on Product / Environment / Arrangement / Equipment — pressure above ~100 psig means balanced seal design.

- Dry gas seals are the modern default (>85% of centrifugal compressors): spiral-groove mating ring creates a ~3-micron non-contact running gap via hydrostatic + hydrodynamic force balance. Know tandem / tandem-with-labyrinth / double arrangements and when each is used.

Gears

- Single-helical gearing requires a thrust bearing (API 613/617); double-helical self-cancels thrust but bearings are still recommended.

- Only 10–30% of gear lube oil actually lubricates — the rest is cooling, sprayed into the outmesh side.

- Manufacturing sequence: hob (generate form) → shave or grind (finish) for high-speed gearing.

Standards map

- Gears → AGMA (+ API 613)

- Rotating equipment/compressors → API 617

- Mechanical seals → API 682

- Aerospace bearings → SAE AS-series

- Performance testing → ASME PTC 22

Exercises

1. (Multiple choice) What percentage of centrifugal gas compressors manufactured today use dry gas seals instead of oil-film seals?
A) ~40%
B) ~60%
C) ~85%
D) ~99%

2. (Open-ended) Explain the physical mechanism that creates the running gap in a spiral-groove dry gas seal — what forces are involved, and why does the gap stay stable during operation?

3. (Open-ended) Why does API require a thrust bearing on single-helical gear sets but only recommend one for double-helical sets? What's happening mechanically in each case?

4. (Open-ended) A tilting-pad journal bearing offers the best resistance to half-speed (oil) whirl among journal bearing types, but it isn't always the default choice. What trade-offs does it bring, and what factors would make an engineer choose a simpler design (like a pressure-dam or offset-halves bearing) instead?

5. (Open-ended) You're asked to choose between a tandem dry gas seal and a tandem dry gas seal with intermediate labyrinth for a compressor handling hydrogen. Which would you pick, and why does the gas being handled matter to that decision?